# CNC Maintenance Knowledge Base for RAG Testing

# CNC Overheating Causes

- Insufficient coolant flow due to clogged nozzles, low coolant level, or pump degradation.

- Overloaded spindle caused by aggressive cutting parameters, dull tools, or improper feed/speed settings.

- Poor ventilation in electrical cabinets leading to servo drive or controller overheating.

- Lubrication failure increasing friction in guideways, ball screws, or spindle assemblies.

- Ambient shop temperatures exceeding equipment operating limits.

# Spindle Vibration Troubleshooting

- Check tool holder balance and confirm correct tool clamping torque.

- Inspect spindle bearings for wear, noise, or temperature rise.

- Verify workpiece clamping rigidity and fixture stability.

- Review cutting parameters for chatter-inducing combinations.

- Measure spindle runout using a dial indicator.

# Lubrication Failures

- Blocked lubrication lines causing dry motion components.

- Incorrect lubricant grade leading to poor viscosity performance.

- Pump malfunction or timer misconfiguration in automatic lubrication systems.

- Contaminated lubricant causing abrasive wear.

# Coolant Flow Issues

- Clogged filters reducing pressure and coolant delivery.

- Air lock in coolant lines.

- Pump impeller wear or motor failure.

- Incorrect coolant concentration causing residue buildup.

# Preventive Maintenance Checklist

- Inspect coolant levels and concentration.

- Check lubrication reservoir and automatic lube cycle operation.

- Clean chip conveyors and machine enclosure.

- Inspect spindle taper cleanliness.

- Check pneumatic pressure levels.

- Review alarm logs for recurring faults.

- Inspect belts, seals, and cable wear.

# Alarm Troubleshooting

- Servo overload alarms may indicate binding axes or excessive acceleration.

- Spindle overheat alarms often correlate with cooling or bearing issues.

- Limit switch alarms can indicate sensor failure or overtravel conditions.

- Communication alarms may stem from loose control wiring or network instability.

# Bearing Wear Symptoms

- Increased vibration and abnormal noise during spindle rotation.

- Temperature rise during extended operation.

- Surface finish degradation on machined parts.

- Increased spindle runout and positional inaccuracies.